OPET

Open PV Electrical Tool

PCB manufacturing Notes

OPET REV 1.4

May 2, 2023

Version 0.93 first draft

Contact Byron McDanold (NREL) in case of questions:

byron.mcdanold@nrel.gov  303-275-4706

Table of Contents

1	Introduction	1

2	PCB manufacturing requirements	1

3	PCB Assembly requirements and instructions	2

3.1	Component orientation	2

3.1.1	Orientation definitions	2

3.2	Through-hole components with heatsinks	5

3.2.1	All OPET versions	6

3.2.2	Low-Current OPET version	6

3.2.3	High-Current OPET version	6

# Introduction

This document contains further information with regards to the PCB design and PCB manufacturing of OPET boards

OPENT comes in 2 versions: low-current (340 mA) and high-current (15 A) version

  - Both use the same PCB with some changes in components

Includes:

  - PCB manufacturing requirements

  - PCB assembly requirements and instructions

## Overview Low-Current OPET version

Figure 1: Photo of previous test version of low-current OPET, the current version is slightly different

## Overview High-Current OPET version

Figure 2: Photo of previous test version of high-current OPET, the current version is slightly different

# PCB manufacturing requirements

The OPET board is a 4-layer PCB

Following copper layer thickness is recommended:

  - top and bottom layer 1 and 4

    - 1oz copper thickness (~35 µm)

  - inner layers 2 and 3:

    - 2oz copper thickness (~70 µm)

following 4-layer PCB structure should be used (see Figure 1):

  - the full PCB should be 1.6mm thick

  - the layers 1 to 2 and 3 to 4 should be separated with a thin prepreg sheet (~0.2 mm)

  - the layer stack should have a centre core sublayer (~1.0 mm) between the inner layers 2 & 3

    - high voltage parts of the OPET PCB are separated such that the inner core sublayer is the main insulator where tracks with high voltage potential cross

Figure 3: recommended PCB layer stack configuration, use 2oz of copper thickness for inner layers and 1oz of copper thickness for outer layers

PCB material:

  - FR4 TG155 or similar

      - good high temperature resistance is needed

PCB finish:

  - should be ENIG (Electroless Nickel Immersion Gold)

# PCB Assembly requirements and instructions

## Component orientation

Before PCB assembly correct orientation of components on the PCB should be checked

  - orientation has to be correct for assembly with a pick and place machine to be successful

  - a component placement preview image/diagram from the manufacturer should help make sure orientation is correct before assembly starts

following sections detail the zero-orientation various component package types

### Orientation definitions

- Zero rotation as shown in the figures

- 2 pin diodes:

  - pin 1, left: C – Cathode

  - pin 2, right: A – Anode

  - zero rotation has pin 1 on left side

Figure 4: 2-pin diode, zero rotation, pin 1 shown on the left side

- 2 pin polarized capacitor:

  - pin 1, left: -, negative

  - pin 2, right: +, positive

  - zero rotation has pin 1 on left side

Figure 5: 2-pin polarized capacitor, zero rotation, pin 1 shown on the left side

- 2 pin electrolytic capacitor:

  - pin 1, left: -, negative

  - pin 2, right: +, positive, also marked with the (+) sign

  - zero rotation has pin 1 on left side

Figure 6: 2-pin polarized electrolytic capacitor, zero rotation, pin 1 shown on the left side

- 44-lead TQFP:

  - Pin 1 marked with extended line on silkscreen

  - Zero rotation has pin 1 in lower left corner

Figure 7: 44-lead TQFP, zero rotation, extended line on pin 1 shown in the lower left corner

- SOIC, µMax, TSSOP and similar ICs:

  - Pin 1 marked with extended line on silkscreen

  - Zero rotation has pin 1 in lower left corner

Figure 8: SOIC, µMax, TSSOP and similar ICs, zero rotation, extended line on pin 1 shown in the lower left corner

- SOT (the exception):

  - Pin 1 marked with extended line on silkscreen

  - Zero rotation has pin 1 in lower right corner

Figure 9: SOP, zero rotation, extended line on pin 1 shown in the lower right corner

- Relays, DIL, DIP:

  - Pin 1 marked with notch in line on silkscreen

  - Zero rotation has pin 1 in lower left corner

Figure 10: Relays, DIL, DIP, zero rotation, notch marking near pin 1 shown in the lower right corner

- Through-hole SIL Connectors:

  - Connector cable input direction / plug entry on open side of silkscreen shape

  - Zero rotation has pin 1 on the left side and open side of silkscreen on the bottom

Figure 11: Through-hole SIL Connectors, zero rotation, cable input direction / plug entry from the open shape side, pin 1 on left side

- SIL components, SIL headers:

  - Pin 1 marked with dot

  - Zero rotation has pin 1 and dot marking on the left side

Figure 12: SIL components and headers, zero rotation, pin 1 on left side marked with dot

- Through-hole header connectors:

  - Pin 1 marked with dot

  - Zero rotation has pin 1 and dot marking on the lower left side, notch marking pointing towards the bottom

Figure 13: Through-hole header connectors, zero rotation, pin 1 on lower left corner marked with dot and notch marking pointing towards the bottom

## Through-hole components with heatsinks

Components with heatsinks have multiple parts on the same component identifier with an additional letter at the end (for example VR2_A for the voltage regulator and VR2_B for the heat sink)

This section details how to mount components with heatsinks onto the OPET board and where insulating pad or thermal paste is needed

some instructions differ between high and low current OPET versions

When soldering through hole components with large heatsinks to the PCB:

  - make sure there is a ~1.5 mm gap between the heatsink/component body and PCB

  - This is to make sure that the heatsink is not contacting and shorting tracks on the PCB and to maintain voltage rating

  - solder the heatsink locating pins to the PCB to stop the component from moving or vibrating

When instructed to use a TO-220 electrical insulating pad:

  - The part then needs to be mounted with an electrically isolating but thermally bonding pad and nylon bush

  - a TO-220 insulating kit such as Newark 50P9593 (MK3306) should work

  - Use a thin pad with high thermal conductivity

  - Max voltage to insulate is 150 V under normal operating conditions

  - The nylon bush is needed to make sure the screw does not make a new contact

  - Make sure the nylon bush has a deep enough rim to go through the component mounting hole and partly into the heatsink screw hole

    - A nylon bush rim that is too shallow will cause a short between component and heatsink

  - Use metal washers at screw head and nut end to spread the contact force on the nylon bush

### All OPET versions

VR2:

  - the 5V regulator should be mounted onto the heatsink without any electrically insulating pads or thermal paste

  - bend the heatsink fins as needed to secure the TO-220 voltage regulator

  - If a hole for screw mounting is available, secure part using M3 screw, nut and washers

Figure 14: Image of VR2 on OPET PCB, the actual heatsink may vary slight and will be either clamp-fit or screw-fit

### Low-Current OPET version

Figure 14: Low-current OPET: Left: front side of T1 with driver MosFET; Right: rear side of T1 with NTC sensor bolted to the back of heatsink and T4 MosFET fixed to small heatsink using either clamp-fit or screw-fit as required

T1:

  - The driver MosFET needs to be mounted with insulating pad and bush

  - In addition, the NTC sensor needs to be mounted to the heatsink

  - The NTC sensor should be mounted at the back of the heatsink, on the opposite side of the MosFET

  - The NTC leads need to be wire soldered onto J16

  - Solder the heatsink with MosFET with a 1.5 mm gap to the PCB as detailed previously

  - the heatsink location pins should be soldered to the PCB to provide best stability

T4:

  - The overcurrent bypass MosFET does not require any additional thermal pads when mounting to the small heatsink

  - Due to the thicker MosFET body it may be required to bend some of the heatsink fins to ~45° before it can be inserted

  - If a hole for screw mounting is available, secure part using M3 screw, nut and washers

### High-Current OPET version

Figure 14: High-current OPET: Left: front side of T4 and RS1; Right: rear side of T4 and RS1; both heatsinks have a component screw mounted on both sides of the heatsink with electrically insulating pats and bushes or thermal paste, as detailed below

T4:

  - The overcurrent bypass MosFET and bypass diode must be mounted together onto the heatsink

  - the MosFET must be mounted with a thermal pad as detailed at start of the main section

  - if the diode has an insulated body with a plastic base, it needs thermal paste to provide a good thermal contact to the heatsink

  - if the diode has a metal base, it needs an insulating pad and bush

  - solder the heatsink with components onto the PCB with a ~1.5 mm gap between the heatsink body and PCB

  - the heatsink location pins should be soldered to the PCB to provide best stability

RS1:

  - The 15A shunt resistor needs to be mounted onto the heatsink together with the switching MosFET

  - To mount the components onto the heatsink a new screw hole must be drilled first

    - This needs to be 4 mm below the existing screw hole or 15mm from the bottom of the heatsink body excluding the locating pins

    - Deburr the newly drilled hole and make sure the contact area is flat and clean

    - The existing screw hole is too far from the edge for the short shunt resistor pins to reach through the PCB with a 1.5 mm gab between the PCB and heatsink

  - The MosFET must be mounted with an insulting pad as detailed at start of the main section

  - The Shunt resistor, if using standard BOM part, has a metal base that is insulated from all contact pins and hence should be mounted with high quality thermal paste

    - High quality meaning high thermal conductivity

  - the heatsink location pins should be soldered to the PCB to improve stability